FMRI STIMULUS AND MONITORING EQUIPMENT

Both 1.5T and 3T research scanners are equipped with stimulus delivery and monitoring systems for fMRI research. These include Epson 8100 3-LCD projectors with Buhl long-throw lenses for rear-view/rear-projection onto Mylar screens. Video signals are carried into the magnet room using a Lightwave FiberLynx optical-fiber VGA connection. Both the projector and the FiberLynx units are housed in custom RF shield boxes with filtered power receptacles. Images are viewed through mirrors mounted on the head coils.

Audio stimuli may be presented via air-conduction headphones available on both the Siemens Sonata and Trio 1.5T and 3T systems, or alternatively via Avotec audio systems or a custom earbud headphone system. An Avotec fiberoptic LCD goggle system with eye-tracking capabilities is also available for use. Most recently, electrophysiologic recording has been added through the support of the Department of Psychiatry's Brain Behavior Laboratory. MRI-safe prescription glasses (which are compatible with all head coils) are available.

Responses are monitored using FORP fiberoptic button boxes installed at 1.5T and 3T, and a Cedrus response pad system is available on the 3T system. Low pass filters in the magnet room penetration panels allow custom interfacing of other stimulus and monitoring equipment using DB-9 connectors.